POSITION TOOLS

There is a submenu indicator on this menu, the three submenus are:

1.  Auto Position

2.  Move Tool

3.  Min. Distance

AUTO POSITION

This function automatically repositions the defined tools according to the user-defined draw type tool clearance and the stroke direction.  See Section 9.1 ANALYSIS SETUP to change the default setting.

MOVE TOOL

This function allows the user to translate any tool in any current C.S direction. See the Figure 9.3.1 and 9.3.2

Figure 9.3.1 Global C.S is Current C.S         Figure 9.3.2 Local C.S is Current C.S

- Select a defined tool from the tool list.

- Select the desired translational direction. The direction with the checkmark is current.

- Enter the number in the DISTANCE field.

- Select APPLY to move the tool in the selected direction the input distance. If APPLY is selected more than once, it will repeat the move.

- Select REVERSE to move the tool in the reverse direction the input distance.

- Select OK to close the window.

MIN (IMUM) DISTANCE

This function allows the user to measure the minimum distance between two selected tools, see Figure 9.3.1.

Figure 9.3.3 Min. Distance

- Select the button to the right of FIRST TOOL or SECOND TOOL. The SELECT TOOLS dialog window appears.

- Select a desired direction. X, Y or Z direction if global C.S is current C.S and U, V, or W direction if the LCS is the current C.S.

- After both tools are selected, click OK to see the minimum distance.

- Select CANCEL to close the window.